# BAM 2.0 / ARAM Provisioning SOP (Sample)

Oct 5, 2026 · @dom

## 1. Purpose & scope

This SOP defines how Production and AMG provision one ARAM and its BAM 2.0 units so every asset reports to ACS and IoT4Bos. It replaces the BAM 1 provisioning steps for BAM 2.0 deployments.

- In scope: ARAM Dust manager and HL78 modem setup, BAM 2.0 Dust mote configuration, ACS and IoT4Bos registration, field verification.

- Out of scope: firmware development, DFU/firmware updates, hardware repair.

- Audience: Production technicians, AMG provisioning staff.

## 2. System overview

Each BAM 2.0 talks only to its ARAM, over the Dust mesh. The ARAM forwards data to the backend over LTE. One ARAM serves multiple BAM 2.0 units.

| Component | Contains | Talks to | Key provisioning IDs |
| --- | --- | --- | --- |
| BAM 2.0 | One Dust mote | ARAM Dust manager (Dust mesh) | Mote address (MAC), network ID, asset ID |
| ARAM | Dust manager, HL78 LTE modem, iMX93 host | BAM 2.0 motes (Dust mesh); ACS (REST API over LTE) | ARAM serial, Dust manager network ID, join key, modem IMEI, SIM ICCID |
| ACS | Backend service | ARAM (REST API) | ARAM record, BAM 2.0 records by mote address |
| IoT4Bos | Asset monitoring platform | ACS | Asset records linked to BAM 2.0 |

Rule: every BAM 2.0 mote must use the same network ID (and join key) as the ARAM it reports to. Each ARAM on a site must have a unique network ID.

## 3. Prerequisites

Equipment

- ARAM unit, powered, with activated SIM installed

- BAM 2.0 units for the site, charged/powered

- PC with USB-UART adapter (3.3 V) and serial terminal (115200 8N1, TBD)

- Label printer or asset tags

Software and access

- Provisioning tool / CLI: [TBD]

- ACS account with device-registration rights (REST API token)

- IoT4Bos account with asset-creation rights

Information needed before starting

- Site name and customer account

- Network ID assigned to this ARAM (from the site network-ID register)

- Asset list: which BAM 2.0 goes on which asset

## 4. Provisioning record

Fill one record per ARAM. Keep it with the site file and attach it to the ACS entry.

ARAM

| Field | Value |
| --- | --- |
| Site / customer |  |
| ARAM serial number |  |
| Dust manager network ID |  |
| Dust join key (stored in [TBD], not on paper) |  |
| HL78 IMEI |  |
| SIM ICCID |  |
| ACS device ID |  |
| Provisioned by / date |  |

BAM 2.0 units on this ARAM

| # | BAM 2.0 serial | Dust mote address (MAC) | Network ID | Asset ID | IoT4Bos asset | Joined (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

## 5. Procedure A: ARAM provisioning

Provision the ARAM first; BAM 2.0 motes cannot join until its Dust manager is configured.

- Record the ARAM serial number, HL78 IMEI and SIM ICCID in the provisioning record.

- Look up the next free network ID in the site network-ID register and reserve it for this ARAM.

- Connect the PC to the ARAM service UART and open the provisioning tool.

- Set the Dust manager network ID and join key.

  - Command: [TBD]

  - Read the values back and confirm they match the record.

- Confirm the HL78 modem registers on LTE.

  - Check: [TBD, e.g. registration status and signal quality]

  - Expected: registered, signal above [TBD].

- Confirm the ARAM reaches ACS (REST API heartbeat or test call).

  - Expected: ACS shows the ARAM online within [TBD] minutes.

- Reboot the ARAM and repeat step 4's read-back to confirm settings persist.

## 6. Procedure B: BAM 2.0 provisioning

Repeat for each BAM 2.0 assigned to this ARAM.

- Record the BAM 2.0 serial number.

- Read the Dust mote address (MAC) from [TBD: device label, CLI, or manager join log] and record it.

- Connect to the BAM 2.0 service UART.

- Set the mote network ID and join key to match the ARAM.

  - Command: [TBD]

  - Read back and confirm both match the ARAM record.

- Assign the asset ID and attach the asset tag to the BAM 2.0.

- Power-cycle the BAM 2.0 within range of the ARAM.

- On the ARAM, confirm the mote shows as joined/operational.

  - Check: [TBD, Dust manager mote list]

  - Expected: mote address listed, state operational, within [TBD] minutes.

- Mark "Joined = Y" in the provisioning record.

## 7. Procedure C: ACS and IoT4Bos registration

ACS (REST API)

- Register the ARAM in ACS using its serial number and IMEI (endpoint: [TBD]).

- Register each BAM 2.0 under that ARAM, keyed by Dust mote address.

- Confirm ACS lists the ARAM with the correct number of child BAM 2.0 units.

- Record the ACS device ID in the provisioning record.

Sample request (illustrative only; replace with the real ACS schema):

POST /api/[TBD]/devices
{
  "aramSerial": "ARAM-000123",
  "imei": "35xxxxxxxxxxxxx",
  "networkId": 1201,
  "bams": [
    { "bamSerial": "BAM2-000456", "moteMac": "00-17-0D-00-00-xx-xx-xx", "assetId": "CYL-7781" },
    { "bamSerial": "BAM2-000457", "moteMac": "00-17-0D-00-00-yy-yy-yy", "assetId": "CYL-7782" }
  ]
}

IoT4Bos

- Create or select the asset record for each asset ID.

- Link each asset to its BAM 2.0 (by serial or mote address, [TBD]).

- Record the IoT4Bos asset reference in the provisioning record.